Software Requirements Specification (SRS)

## Urban Traffic Simulation Engine

Author: Raghvendra Pratap Singh
Version: 1.0 
Purpose: Infrastructure planning and traffic analysis software

1. Introduction

## 1.1 Purpose

The Urban Traffic Simulation Engine is a microscopic traffic simulation platform designed to assist infrastructure planners, transportation engineers, and government agencies in analyzing traffic flow and evaluating road network performance.

The system simulates individual vehicle behavior on road networks and generates analytical metrics such as congestion levels, traffic flow, and road capacity. The platform enables planners to evaluate infrastructure decisions before implementation.

The simulation core is implemented in C++ and designed to integrate with Unreal Engine for visualization.

1.2 Scope

The system models traffic at the vehicle level and supports:

- Road network representation using graph structures

- Lane-level traffic dynamics

- Driver behavior modeling

- Congestion formation and detection

- Dynamic vehicle routing

- Infrastructure performance analysis

The engine is intended for urban traffic analysis, infrastructure planning, and transportation research.

# 2. System Overview

The simulation engine consists of several main components.

### Road Network Module

Represents the road network as a directed graph containing nodes (intersections) and edges (road segments).

### Vehicle Simulation Module

Models individual vehicle behavior including:

- Position and speed

- Lane position

- Driver behavior parameters

- Route destination

- Vehicle movement follows microscopic traffic models.

### Traffic Dynamics Module

Simulates traffic behavior using established traffic flow models including:

- Intelligent Driver Model (IDM) for vehicle acceleration

- MOBIL model for lane-changing decisions

### Metrics and Analysis Module

Measures system-level traffic performance including:

- Throughput

- Average travel time

- Active vehicle count

- Congestion detection

### ScenarioRunner Module

Supports experimental simulation runs including demand sweeps and infrastructure scenario testing.

# 3. Functional Requirements

The system shall support the following functionality:

- Road Network Modeling

- The system shall represent road networks as nodes and edges with multi-lane support.

- Vehicle Simulation

- Each vehicle shall maintain position, speed, lane index, route path, and driver behavior parameters.

- Driver Behavior Modeling

- Vehicles shall be assigned randomized driver profiles such as aggressive, normal, and conservative drivers.

- Car-Following Model

- Vehicle acceleration shall follow the Intelligent Driver Model (IDM).

- Lane Changing

- Lane changes shall follow a simplified MOBIL decision model.

- Dynamic Routing

- Vehicles shall compute routes using the A* pathfinding algorithm and may reroute during simulation.

- Congestion Modeling

- Traffic congestion shall emerge from lane capacity limits and density-dependent speed reductions.

- Metrics Collection

- The system shall record traffic metrics including density, flow, and average speed.

# 4. Non-Functional Requirements

- Performance

- The system shall support simulations of 200–1000 vehicles simultaneously.

- Determinism

- Simulation results shall be reproducible under identical parameters.

- Scalability

- The architecture shall support expansion to city-scale road networks.

- Extensibility

- The system shall allow future integration of traffic signal control, autonomous vehicle models, and smart infrastructure systems.

# 5. External Interfaces

### Map Input

The system shall support importing road networks from OpenStreetMap or GIS datasets.

### Visualization

The simulation shall integrate with Unreal Engine for real-time traffic visualization.

### Output Data

The system shall generate:

- Traffic flow metrics

- Congestion statistics

- Capacity estimates

- Fundamental diagrams

# 6. Future Enhancements

Planned extensions include:

- Shockwave detection and propagation analysis

- Capacity drop modeling

- Monte Carlo traffic simulations

- Large-scale map ingestion

- Infrastructure scenario comparison tools